HeadUnit OS

Project Handoff Document

Automotive Android Auto / CarPlay Head Unit — Software, Prototyping Hardware, and Production Carrier Board

Prepared: August 14, 2026

# 1. Project Overview

HeadUnit OS is an open-source, automotive-grade touchscreen head unit built in Flutter, running on Linux (Raspberry Pi today; a custom RK3566/RK3568 carrier board in development). It replaces a factory or aftermarket car stereo with a 10.1" touchscreen running wireless Android Auto and Apple CarPlay, multi-target audio routing, an FM transmitter fallback, and ignition-aware safe shutdown so the storage medium doesn't corrupt when the key turns off.

The project has two parallel hardware tracks that share the same Flutter application and Linux system layer:

- Raspberry Pi build (shipping today) — a DIY stack: RPi 4/5 + HDMI touchscreen + automotive power HAT + USB audio DAC + optional wireless CarPlay/Android Auto dongle. This is the reference/prototyping platform, and the one a builder can assemble right now from off-the-shelf parts.

- Production carrier PCB (in development) — a custom 6-layer board built around a Rockchip RK3566/RK3568 System-on-Module in a 200-pin SODIMM socket. It integrates everything the RPi build bolts together as separate modules — power protection, ignition sensing, display driver, audio codec + ground isolation, FM transmitter, and USB — onto one board, aimed at mass production.

# 2. Goals

- Ship a head unit that boots on ignition, shuts down cleanly on key-off, and never corrupts its storage — the failure mode that kills most DIY car-Pi projects.

- Support wireless Android Auto and CarPlay both natively (no dongle, $0 hardware) and via a Carlinkit hardware bridge, auto-detected.

- Route audio to AUX/DAC, car Bluetooth, FM transmitter, or HDMI, switchable from the UI.

- Keep the Raspberry Pi build as a low-cost, buildable-today reference platform for individuals.

- Move to a custom carrier board for anyone who wants to manufacture the head unit at volume — same software, purpose-built hardware, single-board integration instead of a stack of HATs and dongles.

# 3. Hardware Track A — Raspberry Pi Prototyping Build

This is the reference build documented in docs/hardware_shopping_list.md and docs/hardware_assembly_guide.md. It is fully specified and buildable; no open engineering items.

## 3.1 Core hardware

| Component | Role | Est. Price |
| --- | --- | --- |
| Raspberry Pi 4 (4/8GB) or Pi 5 | Compute | $55 – $80 |
| 10.1" LCD + HDMI driver board, capacitive touch | Display | $32 – $45 |
| Automotive power HAT (CarPiHAT / StromPi 3 / Mausberry) | 3-wire ignition power + safe shutdown | $20 – $40 |
| 32/64GB microSD, A2-rated | Storage | $8 – $12 |
| USB 3.5mm Hi-Fi DAC + ground loop isolator | Audio out, alternator-whine-free | $15 – $22 |
| Carlinkit CPC200-CCPA wireless dongle (optional) | Hardware CarPlay/Android Auto bridge | $38 – $48 |
| USB mini microphone | Voice assistant input | $6 – $10 |
| Universal 10.1" double-DIN dash mount | Mechanical | $12 – $20 |

Full sourced parts list with AliExpress links: docs/rpi_build_parts_list.xlsx — required items subtotal $168–$261, all items (incl. optional CarPlay dongle and FM transmitter) $221–$329.

## 3.2 Power & safe shutdown

Three wiring options are documented, in order of recommendation: (A) intelligent power HAT with constant-12V + switched-ACC + GND, giving true ignition-triggered boot/shutdown with zero SD corruption risk; (B) a hardwired 12V→5V converter off the ACC fuse with OverlayFS read-only mode enabled as a software safeguard; (C) a 12V cigarette-lighter USB charger, also paired with OverlayFS. Option A is the only one that gives real ignition-aware shutdown; B and C rely on the read-only filesystem to tolerate abrupt power loss.

# 4. Hardware Track B — Production Carrier PCB

Full architecture and rationale: docs/pcb_carrier_board_design.md. Summary below reflects the current state of hardware/pcb/headunit_carrier.kicad_pcb as of this handoff.

## 4.1 Architecture

- Power in: 3-pin automotive harness (BATT constant-12V / ACC switched-12V / GND) → reverse-polarity MOSFET → TVS clamp → buck regulators (12V→5V, 5V→3V3) → SoM.

- Ignition sense: ACC line divider → comparator → RC debounce → SoM GPIO, matching the existing shutdown_listener.py logic; a GPIO-controlled load switch cuts the 5V rail after Linux halts.

- Compute: 200-pin SODIMM socket accepting any pin-compatible RK3566/RK3568 SoM (Radxa CM3S recommended).

- Display: 40-pin 0.5mm FPC connector, 4-lane MIPI-DSI + I2C touch (GT911) + PWM backlight driver — no separate HDMI board.

- Audio: onboard WM8960 I2S codec, isolation transformer ahead of the AUX jack (replaces the inline ground-loop isolator), 3.5mm mic in.

- FM transmitter: Si4713 on the shared I2C bus, SMA antenna connector.

- USB & debug: 2× USB-A host, 1× USB-C OTG/debug, UART header, microSD.

## 4.2 Current build status

| Item | Value |
| --- | --- |
| Board outline | 158.2 × 87.0 mm (≈ 1/7 the area of the first-pass 370 × 275 mm placeholder layout) |
| Layer stackup | 6 layers (F.Cu / GND plane / PWR plane / In3 / In4 / B.Cu) |
| Footprints placed | 45 (41 schematic components + 4 mounting holes) |
| Routing | 526 copper segments, 154 vias, 0 unrouted nets |
| DRC (live KiCad check) | 0 errors, 4 warnings (all cosmetic — mounting-hole footprint not found in project library; no electrical or manufacturing impact) |
| Est. per-unit BOM cost | $104.74 at 100+ unit component pricing (docs/pcb_carrier_board_bom.xlsx) |
| Fab quote (reference) | PCBWay, qty 100, 6L, current size: ≈ $5.03/board landed (PCB + shipping) |

## 4.3 Changes made this session

- Added U9, a VEML7700 I2C ambient light sensor, for automatic backlight dimming — wired into the existing I2C0 bus alongside the FM chip and touch controller; routed on the previously-unused B.Cu layer without touching any of the pre-existing routing.

- Corrected six components flagged by an electrical review: U3 standby LDO (TPS7A02 → TPS7A16-Q1 — the original part was rated for 6V max and was wired directly to the 12V battery rail), U5 load switch (AP2331 → TPS22965 — the original was current-limited to roughly 0.2–0.3A against a 1.5–3A SoM load), D1 TVS diode (SMAJ15A → SMAJ26A, so it doesn't conduct during a 24V jump-start), U1 main buck regulator (TPS54331 → LMR33630-Q1, automotive-rated 36V input for more load-dump margin), U2 (committed to the synchronous TPS62203, dropped the linear AMS1117 alternative), and the ignition debounce capacitor (1µF → 10µF).

- All six substitutions are pin/footprint-compatible, so no net or copper changes were needed — only BOM entries, schematic symbols, and PCB silkscreen text.

- Investigated board size vs. fab cost (see §4.4) and trimmed overlong silkscreen text that was overhanging neighboring parts.

## 4.4 Size / cost investigation (informational — no changes made)

Real PCBWay quotes at qty 100 / 6-layer: current 158.2×87mm size lands at ≈$5.03/unit; a modest, plausibly-achievable tightening to ~140×77mm would land at ≈$4.33/unit (saves ~$0.70/unit); an aggressive 120×70mm — not verified to actually fit the SOM socket and FPC connector — would land at ≈$3.67/unit. PCB + assembly is only 13% of the $104.74 BOM total (the SoM and display panel are 73% of it and don't shrink with board size), so the team decided the current size is fine as-is; no floorplan rework was done.

# 5. Open Decision — USB-C Power Delivery Input

A request came in to make the board's power input a USB-C Power Delivery port instead of (or alongside) the 3-pin automotive harness. This was not implemented — it needs a decision first, because it directly conflicts with the existing ignition-sense architecture in §4.1:

- USB-C PD supplies negotiated voltage (5/9/12/15/20V) but no separate ACC/ignition-switched signal — the current safe-shutdown logic (§2.3 of the design doc) depends on sensing ACC going low when the key turns off, which PD alone cannot provide.

- Option 1: replace the harness entirely, drop true ignition sensing, and trigger shutdown on VBUS loss instead (same comparator concept, different signal — simplest, but behaves differently in a vehicle than a real ACC line).

- Option 2: replace the harness with USB-C PD for power, but add a small second connector or flying lead just for the ACC ignition signal, preserving the existing shutdown logic.

- Option 3: add USB-C PD as a second, alternate input alongside the existing 3-pin harness — full ignition sensing stays intact when running off the vehicle harness; PD is available for bench testing or non-automotive use.

Also implied by any of these: a PD sink controller IC (e.g. a CH224K/STUSB4500-class chip) would need to be added to negotiate a voltage and hand it to the existing buck converter stage, and the input protection stage (TVS rating, fuse rating) would need re-checking against whatever voltage PD ends up delivering.

Next action: confirm which option before any schematic/PCB work starts on this.

# 6. Next Steps Before This Board Goes to Fab

In priority order. Items marked BLOCKER should not be skipped — they will cause a non-functional or unassemblable board.

| Status | Item | Why it matters |
| --- | --- | --- |
| BLOCKER | Swap generic placeholder footprints for real vendor footprints | Every footprint on this board — SODIMM socket, FPC connector, USB connectors, QFN packages, even passives — is an auto-generated generic pad shape based on pin count, not the real part's footprint. This has been flagged since the layout was first generated. Real parts will not solder onto placeholder pads correctly. |
| BLOCKER | Regenerate Gerbers, drill files, and BOM CSV in hardware/pcb/fab_output/ | Everything in that folder predates this session's changes (ambient light sensor, six component substitutions) and does not reflect the current board. |
| OPEN | Decide the USB-C PD power-input question (§5) | Changes the power-input schematic and connector; needs a decision before any layout work starts on it. |
| OPEN | Verify MIPI-DSI differential pairs are routed as true coupled, length-matched pairs | The design doc calls this out as an open item; the routing tool used doesn't inherently do differential-pair routing unless explicitly configured, so this needs a dedicated check in KiCad before signoff. |
| OPEN | Verify trace widths hit target impedance for USB (90Ω differential) and DSI (100Ω differential) | BOM assumes a 'controlled impedance' board, but actual trace widths used were set by net type (power vs. signal), not calculated against the real stackup for impedance targets. |
| OPEN | Confirm the actual display panel FPC pinout once a specific SKU is chosen | Panel vendors are not fully standardized on DSI pinout despite similar connector pitch — called out explicitly in the design doc. |
| OPEN | Mechanical/enclosure check | Connectors that need to reach an enclosure edge (USB, audio jacks, SMA antenna) were placed for routing convenience, not verified against a real enclosure or dash-mount fit. |
| OPEN | Broader electrical review beyond the six parts already corrected | This session's fixes were driven by one external review of five specific parts. Nothing else on the board (inductor saturation current, connector current ratings, codec power sequencing, etc.) has had a dedicated from-scratch check. |
| OPEN | Get formal component quotes | All BOM pricing is estimated at 100+ unit distributor pricing as of Aug 2026, per the BOM's own notes — needs real quotes from Digi-Key/Mouser/LCSC and the SoM vendor before finalizing. |
| PENDING | FCC/CE certification path for the FM transmitter and SoM wireless radios | Explicitly excluded from the current BOM; required for a commercial product, not yet scoped. |

# 7. Reference Documents

| Document | Covers |
| --- | --- |
| README.md | Software feature overview, project structure, build instructions |
| docs/pcb_carrier_board_design.md | Full carrier board architecture, block-by-block design rationale |
| docs/pcb_carrier_board_bom.xlsx | Production BOM, per-block cost breakdown chart, sourcing notes |
| hardware/pcb/headunit_carrier.kicad_pcb / .kicad_sch | Current PCB layout and schematic |
| hardware/pcb/generator/ | Python/kiutils scripts that generate the schematic, symbol library, and PCB footprints/placement |
| docs/hardware_shopping_list.md | Raspberry Pi build parts list and pricing |
| docs/hardware_assembly_guide.md | Raspberry Pi build step-by-step assembly and wiring |
| docs/rpi_build_parts_list.xlsx | Sourced RPi parts list with AliExpress links |
| docs/fm_hardware_guide.md | Si4713 FM transmitter reference (Adafruit breakout on the RPi build; onboard on the carrier board) |